### CPTED Guidelines

General CPTED guidelines for Subdivision:

- Natural Access Control

- Entrances into property should be defined by landscaping, architectural

design, or monitored by a guard

- Landscaping should encourage the use of walkways

- Landscaping in common areas should encourage visibility from residences

- Natural Surveillance

- Landscaping in common areas should encourage visibility from residences

- Parking areas and pedestrian walkways should be well lit

- Lighting for pedestrian walkways should also illuminate surrounding area

- Recreation areas should be visible from a multitude of windows and doors

- Dumpsters  and dumpster enclosures should not create blind spots or hiding areas

- Landscaping should enhance, not inhibit natural surveillance.

- Low shrubbery and fencing should allow visibility from the street

- Territorial Reinforcement

- Use distinctive paving, lawn strips, varying tactile surfaces, ground level changes

to provide physical and psychological barriers.

- Property lines should be defined by landscaping or post and pillar fencing

- Fence supports should face the interior of the property.

- All buildings and residential units should be clearly identified by street address

numbers that are a minimum of five inches high, and well lit at night

- Common doorways should have windows and be key controlled by residents

- Mailboxes should be located near to the appropriate residences.

- Target Hardening

- Single-cylinder dead bolt locks should be installed on all exterior doors

- Door knobs should be 40 inches from window panes

- Exterior door hinges should be located on the interior side of the door

- Windows should be equipped with two locking devices

- Sliding glass doors should have one permanent door on the outside and on the

inside moving door should have a lock device and a pin.

- Management

- Mailbboxes should not identify the unit number or the resident’s name.

Meadow Crest Property Crimes January 2012 thru January 2013:

(with privacy wall)

Burglary Habitation – 1

Burglary Building – 2

Burglary Vehicle – 5

Criminal Mischief – 5

Theft – 1

Adjacent Neighborhood Crimes January 2012 thru January 2013:

(with no wall or other privacy structure)

Burglary Habitation – 12

Burglary Building – 0

Burglary Vehicle – 13

Criminal Mischief – 6

Theft – 6

Pros to having the privacy wall:

- Distinct boundary of if you’re inside the wall walking around you must live in the neighborhood or you will draw attention from neighbors as well as police officers

- Limited access routes in and out of the neighborhood

- Draw less attention from the outside to your personal belongings

- Less trash/debris from major roads like Belt Line and N Garland Avenue

- Public view of a neighborhood that cares and is maintained and probably very close knit as a community

- Protection/safety for children within the neighborhood

- Reduced crime by giving off the appearance of a hard, protected target rather than open and quick

- Reduce cut thru traffic which leads to higher crime, accidents, littering, less community walking due to safety issues, loud music and other noises, and curiosity and opportunity bringing in the wrong type of people